一、Purpose

Sometimes bootcode will crash because user breaks or uses wrong source code and bootcode can’t normally bootup. When you have such problem, you can use recovery tool to help you upgrade the bootcode without unsoldering the flash.

Requirements when you use recovery tool：

- When flash is totally empty, recovery tool can’t be used. If you want to use, at least the first sector of bootcode must exist.

- Recovery tool is located at http://cadinfo/svn/col/DVR/venus/software/system/flash_environment/Bootcode/temp/tools/bootcode_recovery/Debug/rec.exe

- Register setting file is used in Recovery tool, and there are many register setting files for boards.

Currently, we have:
(1) nept.parallel.32x2, nept.parallel.64x1, nept.serial.32x2, nept.serial.64x1
(2) venus.parallel.32x2, venus.parallel.64x1, venus.serial.32x2, venus.serial.64x1
PS: chip_type.flash_type.mem_block_type , please choose the suitable file for your board.
Register setting files are located at
http://cadinfo/svn/col/DVR/venus/software/system/flash_environment/Bootcode/temp/tools/bootcode_recovery/Debug

二、How to use Recovery Tool

- Close all applications that use RS232。

- Power off the target board, and connect your target board to COM1 of PC

- Run the Recovery tool under windows command mode, command: rec
(as picture below)

- Power on the target board, then the recovery tool will send register-value pairs to target board through COM1

- When the transferring is finish, the hint will display. You can input the command again.

- Open the hyper-terminal program, set the com port to COM1. You will see that the regular code is printed in the window.

- Select the menu transfer->send a file then browse to a upgradable bootcode image。Please set the communication protocol to Ymodem-G, after that, press the transfer button to start the upgrade process.

- When the image is transferred completely, please press the cancel button. Then the bootcode will be upgraded automatically.

- When the ‘Finish’ symbole is printed, the bootcode upgrade process is finish. Reset the target board , the target board will boot up again.